Per Use Checklist

Inspected by:  						Date:

Use this general self-inspection checklist every time the laser is to be operated. Report any out of normal issues to the LSO before use.

Each Use Review

| Pre Check | Yes | No | N/A |
| --- | --- | --- | --- |
| Are all authorized users registered with the laser safety program and listed in the SOP? |  |  |  |
| Are all authorized users current with their training? |  |  |  |
| Have the operations and configuration of any authorized lasers changed in a significant manner (e.g., change in layout, configuration, wavelength, etc.)? |  |  |  |
| Are written instructions for using, light maintenance, and setting up the laser equipment kept with the machine? Refer to Manufacturer’s Operation Manual |  |  |  |
| PPE in good order with no tears, cracks, etc. |  |  |  |
| Clothing – Flameproof, fire-resistant outer clothing (wool preferred over cotton), long sleeves, no rolled cuffs, clean of oil/grease; leather aprons/leggings for heavy work. |  |  |  |
| Eyes/Face: Welding helmet and safety glasses with laser shade lens; optical density 7 at a nominal laser wavelength of 1070 nm |  |  |  |
| Hands: Flameproof gauntlet gloves. |  |  |  |
| Feet: Steel-toed boots. |  |  |  |
| Respiratory: Respirator if ventilation is inadequate or fluxes contain fluorine compounds. |  |  |  |
| Equipment Inspection: |  |  |  |
| Set up with proper tips/wire/etc. for the type of material being welded |  |  |  |
| Confirm machine settings to match material type and thickness |  |  |  |
| Gas – correct type and enough to complete task. Tank secured? |  |  |  |
| All hoses and fittings should also be suitable for the application and maintained in good working condition. |  |  |  |
| Emergency Stop button in released mode ready for use |  |  |  |
| Double check safety interlocks operation on door(s) for curtain(s) for proper operation. |  |  |  |
| Postings and Communication |  |  |  |
| Do Class 4 lasers have signs that say you need to wear specific eye protection to use them? |  |  |  |
| Is protective eyewear available and marked with wavelength and optical density? |  |  |  |
| Are laser-controlled areas posted, and equipment labeled with approved signs and labels? |  |  |  |
| Is access to laser controlled area to prevent people being accidentally exposed to the laser beams by posting or controlling the entrance? |  |  |  |
| Safety Conditions |  |  |  |
| Is an inspection covering the items listed below performed prior to each operation? |  |  |  |
| Are windows and ports, which could allow a laser beam to stray into uncontrolled areas covered or protected during laser operation? |  |  |  |
| Are barriers/screens (if present) non-combustible & no burn holes? |  |  |  |
| Is work bench free of unnecessary reflective items? |  |  |  |
| Fire Extinguisher(s) on hand? |  |  |  |
| Removal of any combustibles in the area |  |  |  |
| Inspect Electrical connection, cabling from machine to hand nozzle for tears in protective sheathing. |  |  |  |
| Gas – check hoses, connections, pressure and type |  |  |  |
| Ventilation- Ensure adequate airflow, local exhaust. |  |  |  |
| Shut Down and Clean up |  |  |  |
| Hand laser cleaned up and returned to ‘holster’ |  |  |  |
| Key turned off |  |  |  |
| Gas turned off |  |  |  |
| PPE Stored |  |  |  |
| Check area for any hot spots |  |  |  |
| Have all laser accidents, incidents, or near misses been documented? Follow the company’s APP guidelines for processes in the Safety Statement section |  |  |  |

List any other safety deficiencies found

List corrective actions taken for any identified deficiencies

Other notes

User Request for Change in Operations

Date Submitted: ______________________

Requested by: ______________________

Department: ______________________

Supervisor: ______________________

Current Operation (Baseline)

Laser System (Type/Model): ______________________

Location: ______________________

Current SOP Date: ______________________

Proposed Change

Type of Change: 
☐ Laser parameters
☐ Materials
☐ Location
☐ Fixture/Setup
☐ Personnel
☐ Other: ______________________

- Description of change:

- Reason for Change:

Safety Impact

- Any New Hazards Identified: 
☐ Eye Exposure ☐ Skin Exposure ☐ Fire Risk ☐ Fumes ☐ Electrical ☐ Other: _________

- Preventive Measures:

- Additional PPE/Controls Needed:

Updated Records Checklist

- ☐ SOP

- ☐ JHA Assessment

- ☐ Training Records

- ☐ Per Use Checklist

- ☐ LSO Annual Checklist

Approval

- Requestor Signature: ______________________ Date: __________

- Supervisor Signature: ______________________ Date: __________

- LSO Decision: ☐ Approved ☐ Not Approved

- LSO Signature: ______________________ Date: __________